# ECA13 – MICROWAVE ENGINEERING

# CO1 APPLICATION-BASED ASSESSMENT

ANALYSIS, DESIGN AND VERIFICATION OF A MICROWAVE TRANSMISSION LINE AND NETWORK FOR A 5G MILLIMETRE-WAVE COMMUNICATION SYSTEM

Venkata Keerthi

192412606

Department: Electronics and Communication Engineering Course: ECA13 – Microwave Engineering

Academic Year: 2027–2028 Submitted to: DR.S.KAMATCHI

# ABSTRACT

This assignment presents the analysis and preliminary design of a 28 GHz microwave transmission path for a 5G millimetre-wave front-end. Four transmission structures—coaxial line, rectangular waveguide, stripline and microstrip—are compared in terms of propagation mode, losses, size, integration and cost. A 50 Ω microstrip line on a low-loss substrate is selected for integration with a printed RF front-end. Approximate quasi-TEM parameters are calculated, a loss budget is developed, and a quarter-wave impedance transformer is evaluated for a 75 Ω load. ABCD and S-parameter relationships are used to explain network behaviour. The final design is checked against stated acceptance criteria, including impedance match, insertion loss, mode behaviour, EMI and manufacturing tolerance.

The results are analytical estimates for an undergraduate design study; they are not claimed as measured results or as output from an executed electromagnetic simulator.

# CONTENTS

- Stage 1 – Application and quantified requirements

- Stage 2 – Transmission-structure evaluation and selection

- Stage 3 – Mode and propagation model and dimensions

- Stage 4 – Loss budget and loss-reduction design

- Stage 5 – Network model, matching and S-parameters

- Stage 6 – Validation, critique and tolerance analysis

- Final engineering decision

- References

# DESIGN BASIS AND ASSUMPTIONS

The operating frequency is selected as 28 GHz, within the commonly used FR2 millimetre-wave range. The proposed board uses a low-loss microwave laminate with relative permittivity εr = 3.0, substrate thickness h = 0.254 mm and copper thickness t = 35 μm. The nominal system impedance is 50 Ω. The microstrip calculations use standard quasi-static equations; at 28 GHz, dispersion, copper roughness, launch transitions and frequency-dependent material properties must ultimately be checked using a field solver and measurement.

Important limitation: numerical attenuation values in the loss budget are engineering estimates chosen for demonstrating the method. Exact values depend on the selected laminate datasheet, copper roughness, geometry, connector and electromagnetic simulation. The report distinguishes these estimates from verified results.

# STAGE 1 – FORMULATE THE APPLICATION AND OPERATING REQUIREMENT :

## Application and importance

The selected application is the printed RF feed between a 28 GHz 5G small-cell transceiver and its antenna array. The transmission line carries a modulated RF signal between front-end components. Excessive attenuation wastes transmitter power and reduces receiver sensitivity, while impedance mismatch creates reflections and distorts the delivered power. A controlled-impedance line and compact layout are therefore important.

Microwave engineering developed from Maxwell’s electromagnetic field theory and Hertz’s experimental verification of electromagnetic waves. The cavity magnetron and radar accelerated practical microwave sources and waveguide systems during the twentieth century. Later, satellite links and solid-state microwave devices expanded communication applications. Monolithic microwave integrated circuits (MMICs), printed RF boards and phased

arrays support modern cellular systems, including 5G and developing 6G research. This history explains the shift from bulky waveguide assemblies toward compact, integrated structures where appropriate.

## Frequency-band comparison:

| Band | Example frequency | Free-space wavelength | Advantages | Limitations for this application |
| --- | --- | --- | --- | --- |
| S band | 3 GHz | 100 mm | Lower path loss and easier component design | Larger antennas and components; less available bandwidth in many deployments |
| C band | 6 GHz | 50 mm | Good coverage/penetration trade-off | Antenna elements and passive components larger than at 28 GHz |
| Ka / 28 GHz | 28 GHz | 10.71 mm | Compact arrays and wide channel bandwidth potential | Higher propagation/rain attenuation; layout and material losses become more important |

Wavelength is calculated from λ0 = c/f. At 28 GHz, λ0 = (3.00 × 10⁸)/(28 × 10⁹) = 0.01071 m = 10.71 mm. A 28 GHz band is selected because the objective is a compact millimetre-wave front-end, not maximum long-range coverage.

Rain attenuation and atmospheric absorption depend on weather and exact frequency; the link budget must be separately designed for the deployment environment.

## Quantified requirement specification:

| Parameter | Target / design assumption | Reason |
| --- | --- | --- |
| Application category | Civil telecommunications – 5G small-cell front-end | Compact high-frequency RF link |
| Centre frequency | 28 GHz | FR2 millimetre-wave example |
| Design bandwidth | 27.5–28.5 GHz (1 GHz) | Illustrative engineering target |
| Reference impedance | 50 Ω | Common RF system impedance |
| Line length | 50 mm | Board-level interconnect example |
| Insertion loss | ≤ 1.5 dB over 50 mm, excluding active-device loss | Keeps feed loss limited |
| Return loss | ≥ 15 dB across the target band after matching | Limits reflected power |
| VSWR | ≤ 1.43 corresponding to 15 dB return loss | Equivalent matching criterion |
| Isolation / shielding | Target ≥ 30 dB between sensitive adjacent paths, where applicable | Board-layout EMC target; must be measured |
| Power level | Up to +20 dBm (0.1 W) conducted, design assumption | Moderate small-cell RF feed example |
| Size | Printed line on 0.254 mm substrate; length ≤ 50 mm | Compact integration |
| Cost | Low-cost PCB prototype; budget to be confirmed from local fabrication quote | Cost depends on laminate and fabrication |

## Constraints and acceptance criteria

- EMI/EMC: use a continuous ground plane, short return-current paths, via stitching near launches where appropriate, and avoid long unshielded transitions.

- Safety: +20 dBm is a conducted-power design assumption, not a human-exposure compliance claim. Antenna gain, duty cycle, distance and applicable exposure standards must be assessed for a real product.

- Environment: humidity, temperature, laminate variation and copper roughness can change permittivity and attenuation.

- Acceptance: the line must support the intended signal band, meet the loss and matching targets, and pass tolerance and EMC checks. All targets are preliminary until validated by simulation and measurement.

# STAGE 2 – EVALUATE AND SELECT THE TRANSMISSION STRUCTURE :

## Propagation modes:

| Structure | Mode / field behaviour | Strengths | Limitations at 28 GHz |
| --- | --- | --- | --- |
| Coaxial line | TEM in its fundamental mode; TE11 is the first higher-order mode | Shielded, flexible, well-defined 50 Ω ports | Connector/launch loss and size; higher-order mode limit must be checked |
| Rectangular waveguide | TE10 dominant; higher TE/TM modes above their cutoffs | Low loss and high power handling in suitable bands | Bulky transitions and less convenient direct PCB integration |
| Stripline | TEM when embedded between two ground planes in a homogeneous dielectric | Good shielding and predictable fields | Fabrication and multilayer integration; dielectric/conductor loss |
| Microstrip | Quasi-TEM because fields occupy both substrate and air; small longitudinal field components occur | Compact, low-cost, easy integration with RF chips and antennas | Radiation, dispersion and dielectric/conductor loss; sensitive to layout |

A hollow single-conductor wave guide cannot support TEM because a TEM solution requires both electric and magnetic fields transverse to the propagation direction and a two-conductor potential reference. A hollow wave guide supports TE and TM modes with non-zero cutoff frequencies. Micro-strip is not pure TEM because the dielectric and air regions have different permittivities, so the fields are not entirely confined to a uniform transverse plane.

## Weighted decision matrix:

| Criterion | Weight (%) | Coax score /5 | Waveguide score /5 | Stripline score /5 | Microstrip score /5 |
| --- | --- | --- | --- | --- | --- |
| Frequency / bandwidth suitability | 15 | 4 | 5 | 4 | 4 |
| Low loss | 20 | 4 | 5 | 4 | 3 |
| Power handling | 10 | 4 | 5 | 3 | 3 |
| Size and weight | 15 | 3 | 1 | 4 | 5 |
| Cost | 10 | 3 | 1 | 3 | 5 |
| EMI shielding | 10 | 5 | 5 | 5 | 3 |
| Integration with PCB RF ICs | 20 | 3 | 1 | 4 | 5 |

| Structure | Weighted score / 5 | Interpretation |
| --- | --- | --- |
| Coaxial | 3.65 | Score based on stated integration-focused weights |
| Waveguide | 3.20 | Score based on stated integration-focused weights |
| Stripline | 3.90 | Score based on stated integration-focused weights |
| Microstrip | 4.05 | Score based on stated integration-focused weights |

Decision: micro strip is selected because PCB integration, compactness and cost have substantial weight for a 5G front-end. Wave guide has the strongest low-loss and power-handling advantages, but it is less suitable for a short printed interconnect connected directly to MMIC”s. Coax remains useful for test ports and external connections.

## Sensitivity and strongest alternative

If low loss and power handling were assigned much higher weights while size, cost and chip integration were reduced, wave guide could rank first. If the operating frequency were lowered by a factor of two to 14 GHz, micro strip would remain viable but the electrical dimensions would be larger relative to wavelength and loss could reduce. If frequency were raised to 56 GHz, conductor roughness, dielectric dispersion, launch parasitics and fabrication tolerances would become more severe; a lower-loss substrate and more careful transition design would be necessary. A wave guide alternative becomes attractive where low attenuation or high power matters more than PCB integration.

Design Decision 1: Select 50 Ω micro-strip over rectangular wave guide for compact PCB integration; select wave guide instead if minimum loss and high power dominate.

# STAGE 3 – DEVELOP THE MODE AND PROPAGATION MODEL

## Transmission-line equations

For a uniform line with per-unit-length resistance R, inductance L, conductance G and capacitance C, the telegrapher’s equations are:

𝜕V/𝜕z = −(R + j ω L)I	and	𝜕I/𝜕z = −(G + j ω C)V γ = α + jβ = √[(R + jωL)(G + jωC)]

Z₀ = √[(R + jωL)/(G + jωC)]

For a low-loss line (R ≪ ω L and G ≪ ω C), α ≈ (R/2)√(C/L) + (G/2)√(L/C), β ≈ ω√(LC), and phase velocity v p ≈ 1/√(LC). The load reflection coefficient is Γ = (ZL − Z0)/(ZL + Z0).

## Micro strip geometry and quasi- TEM approximation

Selected substrate assumptions: ε r = 3.0, h = 0.254 mm, copper thickness 35 μ m and nominal Z0 = 50 Ω. The effective permittivity accounts for part of the field travelling through air.

For W/h ≥ 1: εeff ≈ (εr+1)/2 + (εr−1)/2 · [1 + 12h/W]⁻¹ᐟ ² Z₀ ≈ [120π / √εeff] / [W/h + 1.393 + 0.667 ln(W/h + 1.444)]

An iterative design using these quasi-static equations gives a starting width of approximately W = 0.64 mm for 50 Ω (rounded preliminary value). Because the equations are quasi-static and the line operates at 28 GHz, the width must be refined using a field solver and the actual laminate properties.

Worked estimate: W/h = 2.520. Using the stated approximation, εeff ≈ 2.417; vp ≈ 1.930e+08 m/s; λg = vp/f ≈ 6.892 mm; β = 2π/λg ≈ 911.6 rad/m.

## TEM, TE and TM comparison

| Mode | Field components | Cutoff / dispersion | Use in this design |
| --- | --- | --- | --- |
| TEM | Ez = 0 and Hz = 0 | No cutoff in a uniform two-conductor line; non-dispersive in ideal homogeneous media | Ideal reference for coax and stripline |
| TE | Ez = 0, Hz ≠ 0 | Has cutoff; dispersive above cutoff | Waveguide mode, not the intended microstrip mode |
| TM | Hz = 0, Ez ≠ 0 | Has cutoff; dispersive above cutoff | Higher-order waveguide mode |
| Quasi-TEM | Mostly transverse fields; small longitudinal components | Approximate TEM behaviour with frequency-dependent εeff | Appropriate model for the microstrip in its intended operating band |

Microstrip has no simple hollow-waveguide cutoff for its fundamental quasi-TEM mode. Therefore, a conventional TE10 cutoff calculation is not used to define its operating range. The practical single-mode condition is instead checked through a full-wave model for unwanted higher-order substrate, enclosure or feed modes, and by inspecting transitions and ground-plane geometry.

## Propagation parameters and design margin

| Parameter | Preliminary result | Comment |
| --- | --- | --- |
| Substrate relative permittivity εr | 3.0 | Assumed low-loss laminate value |
| Substrate height h | 0.254 mm | Selected geometry |
| Strip width W | 0.64 mm | Starting value; solver refinement required |
| Effective permittivity εeff | 2.417 | Quasi-static estimate |
| Phase velocity vp | 1.930e+08 m/s | c/√εeff |
| Guided wavelength λg | 6.892 mm | At 28 GHz |
| Phase constant β | 911.6 rad/m | At 28 GHz |
| Group velocity | Approx. vp in first-order quasi-static model | Actual value requires dispersion curve |
| Group delay over 50 mm | 0.259 ns | Approximate, neglecting dispersion |
| Mode margin | To be confirmed by EM simulation | No unsupported numerical cutoff claimed for quasi-TEM |

A complete production design should run an eigenmode or full-wave analysis over 27.5–28.5 GHz, including the connector launch and any enclosure. It should confirm that no unwanted mode is excited and that impedance and attenuation remain acceptable at both band edges.

Design Decision 2: The intended mode is quasi-TEM; the preliminary dimensions are W ≈ 0.64 mm and h =

0.254 mm. Dimensions are a starting design, not a validated fabrication release.

# STAGE 4 – EVALUATE LOSSES AND DESIGN A LOSS-REDUCTION STRATEGY

## Skin depth and surface resistance

For copper, using σ = 5.8 × 10⁷ S/m and μr ≈ 1, skin depth δ = √[2/(ωμσ)] ≈ 0.395 μm at 28 GHz. Surface resistance Rs = √(πfμ/σ) ≈ 0.0437 Ω/square. Since copper thickness is much greater than the skin depth, current is concentrated near the surface; surface roughness can increase effective conductor loss.

## Illustrative loss budget

| Loss mechanism | Initial estimate (dB/m) | Improved estimate (dB/m) | Contribution / interpretation |
| --- | --- | --- | --- |
| Conductor | 12.0 | 8.0 | Initial dominant term; smoother copper and wider ground/current paths reduce it |
| Dielectric | 5.0 | 3.5 | Lower-loss laminate / lower loss tangent |
| Radiation | 1.0 | 0.5 | Improved launch and ground-via layout |
| Connector / transition equivalent over 50 mm | 4.0 | 2.0 | Better launch geometry; this is a lumped loss allocated over length for comparison |
| Total equivalent attenuation | 22.0 | 14.0 | Illustrative budget only; requires EM and measurement verification |

For the 50 mm example, line-related loss estimated from the distributed terms (18 dB/m initially) is 18 × 0.05 = 0.90 dB, plus an illustrative connector/transition contribution of 0.20 dB, giving about 1.10 dB. For the improved case, distributed loss is 12 dB/m, giving 0.60 dB plus 0.10 dB transition loss, or about 0.70 dB total. The dB/m table allocates the transition contribution over the 50 mm reference length only to compare budgets; a connector loss is physically a discrete loss.

## Worked conductor and dielectric calculations

For a preliminary loss estimate, conductor attenuation can be represented by αc in Np/m derived from the surface resistance and the current distribution. Exact microstrip expressions depend on strip and ground current geometry; therefore, a precise value should be calculated by an EM solver or a validated microstrip loss model rather than by applying a coax formula directly.

For a low-loss dielectric, the approximate plane-wave dielectric attenuation is αd ≈ (β tanδd)/2 Np/m. Using β ≈ 569 rad/m (rounded from the preliminary geometry) and an illustrative loss tangent tanδd = 0.001, αd ≈ 0.2845 Np/m.

Converting to dB/m using 8.686 gives approximately 2.47 dB/m. This is a simplified dielectric-field estimate; micro strip field filling factor reduces or modifies the effective value and must be included in a more exact model.

## Parametric trends and redesign

- Frequency: conductor loss generally increases as surface resistance rises approximately with √f; dielectric loss often rises with frequency for a fixed loss tangent.

- Conductor: smoother copper, suitable silver/tin finish where appropriate, and avoiding unnecessary narrow sections can reduce conductor loss.

- Dielectric: select a laminate with a lower specified loss tangent at millimetre-wave frequencies; use datasheet values measured at relevant frequencies.

- Roughness: use a laminate/copper foil with controlled roughness and include roughness correction in the simulator.

- Layout: optimize the chip-to-line and connector transitions, maintain a continuous ground reference and use appropriate via fencing.

The illustrative redesign reduces the total equivalent loss from about 1.10 dB to about 0.70 dB over 50 mm, an improvement of 0.40 dB. This meets the Stage 1 target of 1.5 dB in the illustrative budget, but the claim becomes valid only after simulation and measurement.

Design Decision 3: Conductor loss is the largest assumed distributed component. Smoother copper, a lower-loss substrate and optimized launches are selected; the estimated 50 mm loss falls from 1.10 dB to 0.70 dB.

# STAGE 5 – CREATE AND EVALUATE THE NETWORK MODEL

## Equivalent voltage/current and wave variables

For a TEM or quasi-TEM line, voltage and current can be defined through the transverse fields and the conductor geometry. For a hollow waveguide, voltage and current are not uniquely defined in the same way because the fields are not TEM; equivalent modal voltage/current definitions can be introduced by selecting a normalization convention.

a = (V + Z₀I)/(2√Z₀)	b = (V − Z₀I)/(2√Z₀)

Sij = outgoing wave at port i / incident wave at port j, with other ports matched

For a uniform line section of characteristic impedance Zc, electrical length θ = βl and propagation constant γ, an ABCD matrix is:

[A B; C D] = [cosh(γl), Zc sinh(γl); sinh(γl)/Zc, cosh(γl)]

For two cascaded sections, the total ABCD matrix is M total = M1M2. This permits a line and a matching section to be analysed as one two-port network.

## Load mismatch before matching

Assume a real load ZL = 75 Ω connected to a 50 Ω system at the design frequency. This is a simplified load example used to demonstrate matching.

Γ = (ZL − Z₀)/(ZL + Z₀) = (75 − 50)/(75 + 50) = 0.20 VSWR = (1 + |Γ|)/(1 − |Γ|) = 1.5

Return loss = −20 log₁₀|Γ| = 13.98 dB

The 13.98 dB return loss is below the 15 dB target, so the unmatched example does not meet the matching criterion. The reflected power fraction is |Γ|² = 0.04, or 4%.

## Quarter-wave transformer

For a real load, a quarter-wave transformer can match the load to the 50 Ω feed at the centre frequency when its characteristic impedance is Z t = √(Z0 ZL).

Zt = √(50 × 75) = 61.24 Ω

l = λg,t / 4; the transformer electrical length is 90° at 28 GHz

The transformer width must be obtained from the chosen substrate’s impedance-versus-width design equation or field solver. An ideal, lossless quarter-wave transformer gives Γ = 0 at the centre frequency for a purely resistive 75 Ω load. This is a centre-frequency result; over 27.5–28.5 GHz, finite bandwidth, discontinuities and loss cause non-zero reflection.

## S-parameters and interpretation

| Parameter | Meaning | Design interpretation |
| --- | --- | --- |
| S11 | Input reflection coefficient with output matched | Return loss = −20 log10¦S11¦ |
| S21 | Forward transmission coefficient | Insertion loss = −20 log10¦S21¦ for a matched two-port |
| S12 | Reverse transmission coefficient | For a reciprocal passive network, S12 = S21 |
| S22 | Output reflection coefficient with input matched | Indicates output-port matching |

A reciprocal passive microstrip network is expected to have S12 ≈ S21 when both ports use the same reference impedance and the structure contains no non-reciprocal material. A geometrically symmetric network has S11 ≈ S22. A lossy line is not lossless: for a lossless network, the S-matrix is unitary, whereas conductor and dielectric loss make the transmitted power less than the incident power. Good matching means S11 is small; good transmission means

|S21| is close to one.

| Parameter | Before matching | After ideal centre-frequency matching | Target / interpretation |
| --- | --- | --- | --- |
| Z0 / load | 50 Ω system to 75 Ω load | 50 Ω system matched through 61.24 Ω quarter-wave section | Nominal match at 28 GHz |
| ¦Γ¦ | 0.20 | 0 at ideal centre frequency | Lower is better |
| VSWR | 1.50 | 1.00 at ideal centre frequency | Target ≤ 1.43 |
| Return loss | 13.98 dB | Ideally infinite at centre frequency | Target ≥ 15 dB across band; band must be simulated |
| Insertion loss | Depends on line and transition loss | Includes transformer and line loss | Target ≤ 1.5 dB over 50 mm |
| Reciprocity / lossless | Expected reciprocal; lossy | Expected reciprocal; still lossy | S12 ≈ S21; not unitary when lossy |

## Independent verification plan

- Hand-check Γ, VSWR and return loss using the equations above.

- Create a microstrip and quarter-wave transformer in MATLAB, Python/scikit-rf, Qucs-S, ADS or another suitable RF tool.

- Plot S11 and S21 magnitude in dB and phase from 27.5 to 28.5 GHz; mark the 28 GHz centre frequency and acceptance limits.

- Check that S12 ≈ S21 for a reciprocal passive layout and compare S11 with S22 if the network is symmetric.

- Do not label plots as simulation output unless the model has actually been executed. A field solver is required for reliable millimetre-wave launch and dispersion results.

Network decision: the unmatched 75 Ω load fails the 15 dB return-loss target. A 61.24 Ω quarter-wave transformer is the theoretical centre-frequency correction; full-band compliance remains to be verified.

# STAGE 6 – VALIDATE, CRITIQUE AND TOLERANCE ANALYSIS

## Specification verification

| Requirement | Specified | Initial design | Final proposed design (worst case) | Status |
| --- | --- | --- | --- | --- |
| Frequency | 27.5–28.5 GHz | 28 GHz nominal | Must be checked at both band edges | Pending EM verification |
| Mode | Intended quasi-TEM | Microstrip quasi-TEM assumption | Confirm with full- wave/eigenmode model | Conceptually satisfied; verification pending |
| Insertion loss | ≤ 1.5 dB / 50 mm | Illustrative 1.10 dB | Illustrative 0.70 dB | Estimate passes; measurement pending |
| Return loss | ≥ 15 dB | 13.98 dB for 75 Ω load | Ideal transformer matches at 28 GHz | Centre-frequency theory passes; band pending |
| VSWR | ≤ 1.43 | 1.50 | 1.00 ideal at centre frequency | Centre-frequency theory passes |
| Isolation / EMI | ≥ 30 dB where applicable | Not yet measured | Grounding/via fencing proposed | Pending EMC test |
| Power | +20 dBm assumed | Thermal rating not simulated | Check laminate, copper and connector ratings | Pending |
| Size | 50 mm line, 0.254 mm substrate | Meets assumed geometry | Meets assumed geometry | Preliminary pass |

## Tolerance analysis

For a simple first-order estimate, phase constant β is proportional to √εeff and inversely proportional to wavelength. A ±5% width error changes both impedance and effective permittivity; the effect cannot be represented accurately by width percentage alone because Z0 depends nonlinearly on W/h. A practical tolerance study should sweep W by ±5% (0.608–0.672 mm), substrate height by its supplier tolerance and εr by its datasheet tolerance, then recalculate Z0, S11 and insertion loss.

As a sensitivity illustration, if effective permittivity changes by ±3%, vp changes by approximately ∓ 1.5% because vp = c/√εeff. The guided wavelength changes by approximately the same ∓ 1.5% at fixed frequency. This changes the electrical length of the quarter-wave transformer and can degrade the match away from its nominal point.

## Assumptions and their impact

- Ideal conductor assumption ignores skin effect, finite copper thickness and roughness, so it underestimates loss.

- Constant εr and loss tangent ignore frequency, temperature and lot-to-lot dependence.

- Quasi-static microstrip equations do not fully capture 28 GHz dispersion, surface-wave effects, radiation or connector transitions.

- A 75 Ω purely resistive load is a teaching example. Real RF front-end impedances may be complex and vary with frequency.

- The ideal quarter-wave transformer is narrowband relative to a broadband matching network and is sensitive to fabrication errors.

## EMC, safety and experimental test plan

- Inspect ground continuity, connector seating, solder quality and board cleanliness before powering the circuit.

- Use a calibrated VNA with appropriate 28 GHz-capable cables and calibration standards; calibrate at the measurement reference plane.

- Measure S11 and S21 over 27.5–28.5 GHz and compare the results with the target limits.

- Use a spectrum analyzer and suitable near-field probes, where available, to identify unwanted emissions and coupling between adjacent paths.

- Confirm RF power handling and exposure compliance using the complete system configuration and applicable standards; do not infer exposure safety from conducted power alone.

- Repeat measurements on more than one fabricated sample if possible, and record uncertainty, calibration method, substrate lot and ambient temperature.

## Proposed further improvement

If measured loss or return loss fails, first optimize the launch and quarter-wave width in a full-wave simulator, then consider a lower-loss laminate and smoother copper. If the operating bandwidth is too wide for a quarter-wave transformer, evaluate a two-section transformer or a tapered matching section. The improved design should be re-simulated, fabricated and measured before adoption.

# FINAL ENGINEERING DECISION

## Initial design: NO

The initial 50 Ω microstrip connected directly to the illustrative 75 Ω load does not meet the specified return-loss target: the calculated return loss is 13.98 dB and VSWR is 1.50, compared with targets of at least 15 dB and no more than 1.43. The illustrative loss estimate is within the insertion-loss limit, but matching is not acceptable.

## Final proposed design: CONDITIONAL YES

The 50 Ω microstrip with a 61.24 Ω quarter-wave transformer is a suitable preliminary solution for a compact 28 GHz 5G PCB front-end. At the centre frequency, ideal transmission-line theory predicts a match to a 75 Ω resistive load.

The illustrative redesigned loss is about 0.70 dB over 50 mm, below the 1.5 dB target. However, final adoption is conditional on full-band S-parameter simulation, manufacturing-tolerance analysis, power checks, EMC testing and VNA measurement. It would be technically incorrect to claim that the design has passed those tests before they are performed.

Therefore, the final structure and mode should be adopted for prototype development, but not released as a production design until the pending verification tasks demonstrate compliance across 27.5–28.5 GHz.

# REFERENCES

- D. M. Pozar, Microwave Engineering, 4th ed., Wiley, 2012.

- R. E. Collin, Foundations for Microwave Engineering, 2nd ed., IEEE Press/Wiley, 2001.

- C. A. Balanis, Advanced Engineering Electromagnetics, 2nd ed., Wiley, 2012.

- D. K. Cheng, Field and Wave Electromagnetics, 2nd ed., Addison-Wesley, 1989.

- IEEE Std 145-2013, IEEE Standard for Definitions of Terms for Antennas.

- IPC-2141A, Design Guide for High-Speed Controlled Impedance Circuit Boards, IPC.

- Manufacturer datasheet for the selected microwave laminate: use the exact supplier and revision chosen for fabrication.

- 3GPP TS 38.101-2, NR; User Equipment (UE) radio transmission and reception; Range 2 Standalone (consult the applicable release).

# APPENDIX A – FORMULA SUMMARY

| Quantity | Formula |
| --- | --- |
| Free-space wavelength | λ0 = c/f |
| Propagation constant | γ = √[(R+jωL)(G+jωC)] = α+jβ |
| Low-loss phase constant | β ≈ ω√(LC) |
| Reflection coefficient | Γ = (ZL−Z0)/(ZL+Z0) |
| VSWR | (1+¦Γ¦)/(1−¦Γ¦) |
| Return loss | −20 log10¦Γ¦ dB |
| Quarter-wave transformer | Zt = √(Z0 ZL), l = λg/4 |
| Group delay | τg = −d∠S21/dω; approximate l/vg for a low-dispersion line |
| Dielectric attenuation approximation | αd ≈ β tanδd / 2 Np/m for a uniform low-loss dielectric field |
| Np/m to dB/m | α(dB/m) = 8.686 α(Np/m) |

Submission note: Replace the illustrative assumptions with your instructor-approved values and attach actual MATLAB/Qucs-S/ADS or VNA plots if required. Verify all dimensions and loss figures against the selected laminate and actual fabrication stack-up.